Modeling and Numerical Study of Thermoelectric Generators

(Report submitted as part of B. Tech. Final Year Project)

Submitted by 
Soham Banerjee (2021MEB069)

Ishita Chaudhary (2021MEB070)

Pabitra Bhuyan (2021MEB090)
Department of Mechanical Engineering 
Indian Institute of Engineering Science and Technology, Shibpur

Under the Guidance of 
Dr. Bijan Kumar Mondal
Department of Mechanical Engineering 
Indian Institute of Engineering Science and Technology, Shibpur

# ACKNOWLEDGEMENT

We take this opportunity to express our heartfelt gratitude to all those who have supported and guided us throughout this project. First and foremost, we extend our sincere gratitude to our project guide, Dr. Bijan Kumar Mondal, for his invaluable guidance, encouragement, and insights. His expertise in thermal sciences, coupled with his constructive suggestions, has been instrumental in shaping our understanding of the subject and refining our approach at every stage of this work. We are truly fortunate to have benefited from his mentorship.

We also wish to express our sincere appreciation to the Head of the Department, Dr. Subhash Chandra Mondal, for his unwavering support and for fostering an environment conducive to learning and research. Additionally, we are grateful to the faculty and staff of the Department of Mechanical Engineering for equipping us with a strong academic foundation and providing access to the resources necessary for the successful progress of this project. Their encouragement and guidance have motivated us to strive for excellence.

We extend our heartfelt thanks to our peers and colleagues for their valuable discussions, constructive feedback, and enthusiastic support. Their camaraderie and collaborative spirit have made this journey both enriching and enjoyable, inspiring us to work with diligence and creativity.

A special word of thanks goes to our parents, whose unwavering support, patience, and encouragement have been a constant source of strength and inspiration for us. Their belief in our abilities has been instrumental in helping us overcome challenges and remain focused on our goals. We also wish to express our gratitude to our extended family members and friends, who have provided emotional support, motivation, and understanding throughout this endeavour. Their presence has been invaluable, reminding us of the importance of balance and perseverance.

Lastly, we would like to acknowledge the authors and researchers whose published works have provided a solid foundation for our study. Their significant contributions to the field of thermoelectric generator modeling have been a guiding light, helping us navigate the complexities of our project and deepen our theoretical understanding.

This project would not have been possible without the collective efforts, encouragement, and support of everyone mentioned above. We remain profoundly thankful to all those who have played a part in the successful progress of this work.

# ABSTRACT

This study presents a comprehensive modeling and numerical investigation of thermoelectric generators (TEGs), emphasizing their potential to convert waste heat into electrical energy. The research begins with a detailed analysis of the structure and operational principles of TEGs, highlighting the role of thermoelectric effects—Seebeck, Peltier, Thomson, and Joule—in determining system performance. A rigorous mathematical model is developed, incorporating these effects to derive a generalized heat transfer and electrical energy equation. The analytical solutions are systematically obtained for each TEG component, including semiconductor materials, copper plates, and ceramic substrates, under steady-state conditions with specific thermal and electrical boundary conditions.

The model is implemented in MATLAB to perform parametric studies, examining the influence of critical design variables such as the temperature gradient, semiconductor height, external load resistance, and the number of thermoelectric modules on system output. Numerical simulations reveal that increasing the hot-side temperature or the height of semiconductors enhances both power output and efficiency, whereas higher cold-side temperatures and external circuit resistances adversely affect performance. Additionally, increasing the number of thermoelectric module pairs proportionally improves energy generation, indicating scalability.

The findings offer valuable insights into the interdependence of material properties, geometric configurations, and external operating conditions in shaping TEG performance. The study lays a solid foundation for assessing TEG systems and highlights opportunities for future research, such as the development of advanced thermoelectric materials and the integration of dynamic heat sources to improve energy conversion efficiency and operational robustness.

- TABLE OF CONTENTS

1.	Introduction	1

1.1	Literature Review	1

2.	Thermoelectric Generator	2

2.1	Construction	2

2.2	Working Principle	4

2.3	Thermoelectric Effects	4

3.	Modeling	5

3.1	Model Description	5

3.2	Generalised Equation	6

3.3	Analytical Solution	8

3.3.1	Topside Ceramic Plate	8

3.3.2	Topside Copper Plate	8

3.3.3	Semiconductor	9

3.4	Matrix of Constants	11

3.5	Current Generated	12

4.	Result and Discussion	13

5.	Conclusion and Future Work	17

5.1	Conclusion	17

5.2	Future Work	17

6.	References	18

7.	Appendix	20

7.1	MATLAB Code for Parametric Study with respect to Length of Semiconductor	20

7.2	MATLAB Code for Parametric Study with respect to External Circuit Load	22

7.3	MATLAB Code for Parametric Study with respect to Number of Semiconductor pairs	24

7.4	MATLAB Code for Parametric Study with respect to Cold Side Temperature	27

7.5	MATLAB Code for Parametric Study with respect to Hot Side Temperature	29

# Introduction

Access to energy in the right form, at the right time, and at the right place is a premise for our society to continue to prosper. Accordingly, with the stipulated population growth and increase in welfare, the power production must continue to grow, however in appropriate ways [1]. The power supply for a modern society must deal with production, storage and distribution and must come from diverse sources. Moreover, for the society to be sustainable the sources of energy supply must eventually be renewable. Reusing a waste, such as waste heat, is considered a conditional renewable energy source. Furthermore, meeting the future need of power supply requires that we also deal with the concept of energy quality. Electric energy is for instance of greater value than thermal energy. Heat at high temperatures is of greater value than at lower temperatures. Heat on demand is of greater value than intermittent and distributed heat. Thermoelectric generators represent a scalable and robust technology to convert waste-heat to high quality electricity. In this paper, we investigate the implementation of thermoelectric generators in the metal casting industry, where intermittent high-quality heat is available alongside with a power demand.

## Literature Review

Thermoelectric generators (TEGs) convert a thermal potential (temperature difference) directly to an electric potential (voltage) when heat passes through the generators [2]. To our knowledge, this effect was first discovered in the 19th century by Seebeck [3]. The Seebeck effect occurs when two different charge conductors (e.g., p- and n-type semiconductors) are coupled in series via two connections (A-B-A) and when the connections are at different temperatures (A-B at T1 and B-A at T2). As long as the temperatures T1 and T2 deviate, we can measure an electric potential in the circuit that is proportional to the temperature difference, where the slope equals the Seebeck coefficient, a, of the pair of conductors. Thermoelectric generators are scalable systems, ranging from microwatts to kilo-watts applications [4,5]. The efficiency of a TEG relative to the Carnot efficiency is typically below 5%, which is significantly lower than the same relative efficiency of competing technologies such as steam turbine systems and organic Rankine cycle systems, where steam turbine systems can have efficiencies relative to the Carnot efficiency that exceed 40% [6]. The main advantages of TEGs compared to alternative technologies are their size (plates of a few millimetres), scalability, fairly simple constructions (no moving parts), weight (used for mobile applications) and flexibility (fitting to intermittent heat sources) [6]. TEGs can be used in numerous applications, such as waste heat recovery [14] and solar energy operation, experimental measurements of solar thermoelectric generators with a peak efficiency of 9.6% and a system efficiency of 7.4% are reported by Kraemer et al [15]. Bayod-Rujula et al. [16] designed and constructed presented a design and developed of an experimental installation consisting of two Photovoltaic-thermal hybrid solar collectors, one of them with integrated TEG modules. This prototype would make it possible to observe the rise in electrical efficiency that can be accomplished by introducing TEG in PVT modules. Similarly, A novel hybrid energy harvesting (EH) system consisting of a dynamic offset feed mirrored parabolic integrated solar panel with thermoelectric generator (TEG) on the lower side of the solar panel was proposed by Singh et al. [17]. The proposed energy harvesting device simultaneously absorbs both solar and thermal energy, making the system ideal as a hybrid energy harvesting system. More recent investigation into solar thermoelectric generators can be found in references [18,19,20].

An early application was a generator for remote subsea wellheads [21]. One of their strengths is their flexibility in application as noted by the following examples. A multigeneration system combined with a thermoelectric generator based on biomass gasification equipment was introduced in a study [22]. A flexible thermoelectric generator using eutectic gallium indium liquid metal together with a high thermal conductivity elastomer was designed to harvest body heat which can then be used for wearable electronics [23,24]..A triple micro combustor aimed at portable power generation was designed and developed to enhance heat transmission from hot gases to thermoelectric modules considerably [25].

Now there are many materials available for use when it comes to thermoelectric generators. Those most commonly referred to are inorganic, and are applicable for temperatures up to 550 K (BiSnTe-based, BiSbTe-based), up to 800 K (PbTeSe, PbSe-based) or up to 1200 K (SiGe) [6]. One obvious goal in the development of thermoelectric generators is to have a large Seebeck coefficient (a) for the materials. This is observed for conducting organic systems [7,8] as well as for membrane and super-capacitor systems [9-11]. Recently, Børset et al. [12] and Kang et al. [13] reported very high Seebeck coefficients (0.9 mV/K < a < 1.3 mV/K) using molten carbonate cells reversible to carbon dioxide and oxygen for the temperature range above 650 K.

Since thermoelectric generation is an immensely growing sector, we need to have more and more accurate models for thermoelectric generators. In this work we focus on developing a more accurate model of thermoelectric generator considering all the effects i.e. Seebeck effect, Peltier effect, Thomson effect as well as Joule effect and experimentally verify it. This work not only gives detailed insights in the operation of thermoelectric generators but also helps in further exploration in the field.

The paper begins with the discussion of the structure and working principle of thermoelectric generator. It moves on to modeling the thermoelectric module where first a generalized equation is derived and then it is solved analytically for each component. A matrix is obtained as a result of the analytical solution which is further solved in MATLAB. A numerical study is then carried out by varying different parameters. The results of this study is discussed. The paper is thus concluded by highlighting the future work of the project.

# Thermoelectric Generator

The basic construction and working of a thermoelectric generator are explained in the following sections.

## Construction

A thermoelectric generator is made up the following major components as shown in Figure 1:

Figure 2.1: TEG Structure

- Heat Source and Hot Side Heat Exchanger:

The heat source supplies thermal energy to the TEG, creating the temperature gradient essential for its operation. The hot side heat exchanger facilitates efficient heat transfer from the source to the thermoelectric modules. It is typically made from thermally conductive materials such as copper or aluminium, ensuring minimal thermal resistance. Designs often incorporate fins, microchannels, or extended surfaces to maximize heat transfer area and uniformity across the module. The heat exchanger must also withstand high temperatures and thermal cycling without significant deformation or degradation, making materials with high thermal stability crucial.

- Thermoelectric Modules:

The thermoelectric module is the core component of a TEG, directly responsible for converting heat into electricity. Its design involves:

  - p-Type and n-Type Semiconductors: The thermoelectric core uses p-type (hole-dominant) and n-type (electron-dominant) semiconductors because they satisfy critical material property requirements

    - High Seebeck Coefficient (α): This property determines the voltage generated per unit temperature difference, making semiconductors ideal as they exhibit significantly higher values than metals.

    - Low Thermal Conductivity (κ): To maintain a steep temperature gradient, materials must minimize heat leakage.

    - Moderate Electrical Conductivity (σ): Materials must conduct electricity efficiently while balancing Joule heating losses.

These properties make p-n semiconductor pairs optimal, as they enable efficient thermoelectric performance by allowing charge carriers (electrons and holes) to flow in opposite directions, enhancing voltage output.

Arrangement: Alternating n-type and p-type semiconductor blocks are connected electrically in series and thermally in parallel.

Materials:
Bismuth Telluride (Bi2Te3): High efficiency at moderate temperatures.

Lead Telluride (PbTe): Best for high temperatures.

Silicon-Germanium (SiGe): Operates well at extreme temperatures.

  - Interconnects:

Thermoelectric elements are connected in series electrically and in parallel thermally using interconnects made of metals like copper or nickel. These interconnects must have low electrical resistance to minimize power losses and high thermal conductivity to maintain efficient heat flow. The interconnects are soldered or bonded to the semiconductors, requiring materials that can withstand thermal expansion and mechanical stresses during operation.

  - Substrates:

The thermoelectric elements and interconnects are supported by substrates, which provide structural stability while electrically insulating the circuit. Substrates are typically made of ceramics like alumina (Al2O3) or aluminium nitride (AlN), chosen for their high thermal conductivity, low thermal expansion, and excellent insulation properties. They also help distribute heat evenly across the thermoelectric elements, improving module performance.

  - Encapsulation:

The entire thermoelectric module is encapsulated to protect sensitive components from environmental factors such as oxidation, humidity, and mechanical damage. Encapsulation materials, such as thermally stable polymers or glass, ensure durability and long-term reliability

- Cold Side Heat Exchanger:

The cold side heat exchanger dissipates the residual heat from the thermoelectric module into a cooling medium, maintaining the required temperature gradient across the device. Like the hot side, it is typically constructed from thermally conductive materials such as aluminium or copper and designed with features like fins, liquid cooling channels, or heat pipes to enhance heat dissipation. The choice of cooling medium—air, water, or other fluids—depends on the application and cooling requirements. For portable or compact TEGs, air cooling is often preferred, while industrial systems might use active liquid cooling for superior performance. The design must ensure that heat dissipation is both uniform and efficient to maximize the module's thermoelectric conversion efficiency.

- External Circuit and Load:

The external circuit in a thermoelectric generator connect the module to the load, enabling the transfer of generated electrical energy. The load, which may include sensors, batteries, or other devices, determines the utilization of the output power. Power conditioning components, such as voltage regulators or boost converters, are often included to adjust the voltage and ensure compatibility with external systems. These elements are designed to handle fluctuations in output and provide stable operation across varying applications.

## Working Principle

A thermoelectric generator (TEG) works by converting heat into electricity using the Seebeck effect. When one side of the TEG is heated and the other side is cooled, a temperature gradient (ΔT) is established across the thermoelectric module. This gradient causes charge carriers in the thermoelectric materials to diffuse. In the n-type semiconductor, electrons gain thermal energy at the hot side, increasing their kinetic energy and driving them toward the cooler side, where their energy state is lower. Conversely, in the p-type semiconductor, thermal excitation at the hot side creates additional holes, which diffuse toward the cold side. These movements generate a voltage difference across the material. When the circuit is closed, this voltage drives an electric current through an external load, completing the energy conversion process. To maximize efficiency, the thermoelectric elements are arranged thermally in parallel (sharing the same heat flux) and electrically in series (to add voltages). This design ensures that the available thermal energy is utilized effectively while providing a sufficient electrical output. The efficiency of this system depends on the material properties and the magnitude of the temperature gradient.

## Thermoelectric Effects

Several thermoelectric effects take place inside a thermoelectric generator. These are:

- Seebeck Effect:
The Seebeck effect is the phenomenon where a temperature gradient across a conductive or semiconductive material generates an electric potential. This effect is the basis for TEG operation. The relationship between the generated voltage (V) and the temperature difference (ΔT) is expressed using the Seebeck coefficient (S) of the material as [26]: 

S=ΔVΔT #(2.1)

- Peltier Effect:
The Peltier effect occurs when an electric current flows through a junction of two different materials, causing heat to be absorbed at one junction and released at the other. This effect is described mathematically by the Peltier coefficient (Π) [27]:

ⅆQⅆt=Πⅈ #2.2

where Q is the heat absorbed or released, and i is the current. 
The Peltier and Seebeck coefficients are related as [28]:

Π=ST #2.3

- Thomson Effect:
The Thomson effect describes the heating or cooling of a material when an electric current flows through it in the presence of a temperature gradient. The heat generated or absorbed per unit length is given by:
μ=ΔVΔT #2.4

- Joule Heating:
Joule heating, also known as resistive heating, occurs due to the resistance (R) of the thermoelectric materials. The rate of heat generated (Q) by this process is given by [29]:

Q=ⅈ2R#2.5
where i is the current.
While this effect is a loss mechanism in TEGs, it is considered during material and design optimization to minimize inefficiencies.

# Modeling

## Model Description

For modeling purposes, we consider a simple thermoelectric module with one pair of p and n type semiconductors. The semiconductor pair is sandwiched between a layer of ceramics followed by a layer of copper plate. The external load is connected with the copper plates thus completing the circuit. The axis and dimensions used for the model are depicted in Figure 2.

Figure 3.1: Thermoelectric module schematic

In order to simplify the model, a few assumptions have been made. These assumptions are listed below.

- The system is assumed to be in steady state.

- The properties of thermoelectric materials like thermal conductivity, Seebeck coefficient and Thomson coefficient are assumed to be isotropic and homogenous.

- Convection and radiation from the TEG module are neglected.

- The TEG module is considered to be insulated, hence losses are assumed to be zero.

- There is no variation in temperature along x and z directions.

- The electrical energy generated is equal to the loss of heat energy in the TEG module.

- All kinds of other electrical losses are assumed to be zero.

Table 1 lists the notations used throughout this report.

| Symbol | Meaning |
| --- | --- |
| lce1 | Length of top ceramic plate |
| lcu1 | Length of top copper plate |
| lsc | Length of p and n type semiconductors |
| lcu2 | Length of bottom copper plate |
| lce2 | Length of bottom ceramic plate |
| bp | Width of p type semiconductor |
| bg | Width of gap between p and n type semiconductor |
| bn | Width of n type semiconductor |
| kce | Thermal conductor of ceramic plate |
| kcu | Thermal conductor of copper plate |
| kp | Thermal conductor of p type semiconductor |
| kn | Thermal conductor of n type semiconductor |
| αp | Seebeck coefficient of p type semiconductor |
| αn | Seebeck coefficient of n type semiconductor |
| Πp | Peltier coefficient of p type semiconductor |
| Πn | Peltier coefficient of n type semiconductor |
| μ | Thomson coefficient |
| T | Temperature |
| J | Current density |
| E | Electric Field |
| q | Heat Flux |

Table 1: Notation

## Generalised Equation

Consider a differential elemental volume ⅆV=dx dyⅆz at a distance (x, y, z) from the origin as shown in Figure 3.2. The heat flux entering and exiting the volume is depicted by arrows. The rate of change of internal energy inside the volume (Qin) is given by:

Qint=∂ρCpT∂tⅆV #3.1

The amount of heat generated within the volume will is given by:

Qgen=SⅆV #3.2

where S is the rate of heat generated per unit volume.

The rate of heat flow is equal to the product of the heat flux and the area through which the heat is flowing. Thus, the rate of heat flow entering the volume (Qin)  will be given by:

Qin= qx dy dz + qy dz dx + qz dx dy #3.3

The rate of heat flow exiting the volume (Qout)  will be given by:

Qout= qx+dx dy dz + qy+dy dz dx + qz+dz dx dy #3.4

Figure 3.2: Elemental Volume

By conservation of Heat Energy,

(Total rate of heat flow entering the volume – Total rate of heat flow exiting the volume) + Heat generated in the volume = Change in internal energy of the volume

Thus, from Eqs 3.1 to 3.4,

(qx-qx+dx)dy dz + (qy-qy+dy)dz dx+ (qz-qz+dz)dx dy+ S dV= ∂ρCpT∂tⅆV #⇒ (qx-qx+dx)dx+(qy-qy+dy)dy+ (qz-qz+dz)dz+S= ∂ρCpT∂t###

⇒ -∇q+S= ∂ρCpT∂t   #3.5

According to Fourier’s Law:

q=-k∇T #3.6

The rate of heat generated per unit volume is given by:

S=E⋅J #3.7

where E is the Electric Field vector and J is the current density vector.

The Electric field is the Potential gradient and can represented as:

E=-∇V #3.8

The potential gradient is caused by three effects:

- Flow of current:

∇V=-ρJ#3.9

where ρ is the resistivity.

- Seebeck Effect:

From Eq. 2.1, it can be shown that:

∇V=α∇T#3.10

- Thomson Effect:

From Eq. 2.4, it can be shown that:

∇V=μ∇T#3.11

Since the system is assumed to be in steady state, ∂ρCpT∂t=0 #3.12

From Eqs 3.5, 3.6, 3.7 and 3.12, the generalised model can be summarized as:

∇k∇T+E⋅J = 0 #3.13

where on combining Eqns 3.8 to 3.11,  
E= ρJ- α∇T- μ∇T #3.14

## Analytical Solution

In this section, we will carry out a component wise analysis and derive its analytical solution.

### Topside Ceramic Plate

The following points are to be noted with respect to the ceramic plate:

- There is no current flowing through the ceramic plate. Thus J=0.

- Since there is no variation in temperature in x and z directions, ∇T=ⅆTⅆy

Thus, substituting in the generalised Eq. 3.13, we get:

kⅆ2Tⅆy2=0#3.15

On solving Eq. 3.15, the following equations are obtained:

- Tce1=C1,ce1y+C2,ce1                                                                                                                        3.16

- ⅆTce1ⅆy=C1,ce1                                                                                                                          (3.17)

The following boundary conditions can be applied to the ceramic plate (y = 0 to y = lce1):

- The temperature at the top surface will be equal to the hot side temperature:

Tce1y=0=TH #3.18

- Peltier effect between ceramic and copper plates is negligible hence flux leaving the bottom surface of ceramic plate will be equal to the flux entering the top surface of the copper plate:

ⅆTce1ⅆyat y=lce1=kcukce×ⅆTcu1ⅆyat y= lce1#3.19

### Topside Copper Plate

The following points are to be noted with respect to the copper plate:

- In the copper plate, the current flows dominantly in the x direction. Therefore, Jy, Jz are considered to be negligible. Thus J=Jxl.

- Since there is no variation in temperature in x and z directions, ∇T=ⅆTⅆy

- Thomson and Seebeck effects are negligible. Thus, E= ρJ

Thus, substituting in the generalised Eq. 3.13, we get:

kⅆ2Tⅆy2+ρJx2=0#3.20

For simplification, a new coordinate system (x, y’, z) is introduced as shown in Figure 3.1. Here, y’ = 0 lies at y =  lce1.

On solving Eq. 3.20, the following equations are obtained:

- Tcu1=C1,cu1y'22+C2,cu1y'+ C3,cu1                                                                                              3.21

- ⅆTce1ⅆy'=C1,cu1y'+ C2,cu1                                                                                                       (3.22)

where, C1,cu1 = - ρcuJx,cu12kcu                                                                                                             (3.23)

The following boundary conditions can be applied to the copper plate (y’ = 0 to y’ = lcu1):

- The temperature at the top surface of the copper plate will be equal to the temperature of the bottom side of the ceramic plate:

Tcu1y'=0=Tce1y=lce1#3.24

- Peltier effect between ceramic and copper plates is negligible hence flux leaving the bottom surface of ceramic plate will be equal to the flux entering the top surface of the copper plate:

ⅆTce1ⅆyat y= lce1=kcukce×ⅆTcu1ⅆyat y= lce1#3.25

- Due to Peltier effect between copper plate and semiconductor, some heat flux will be lost when heat enters from copper to p-type semiconductor whereas some heat flux is gained when heat enters from copper to n-type semiconductor:

kculp+lnⅆTcu1ⅆyat y'= lcu1= kplpⅆTpⅆyat y'= lcu1+ knlnⅆTnⅆyat y'= lcu1+ ΠplpJy,p-ΠnlnJy,n                                                                                                            (3.26)

### Semiconductor

The following points are to be noted with respect to the semi-conductor:

- In the semiconductor, the current flows dominantly in the y direction. Therefore, Jx, Jz are considered to be negligible. Thus J=Jyj.

- Since there is no variation in temperature in x and z directions, ∇T=ⅆTⅆy

Thus, substituting in the generalised Eq. 3.13, we get:

ⅆ2Tⅆy2-μJy+αJykⅆTⅆy+ρJy2k=0#3.27

It can be expressed in the form:

ⅆ2Tⅆy2+AⅆTⅆy+B=0

where,  A=-μJy+αJyk                                                                                                                                      (3.28)

B=ρJy2k                                                                                                                                       (3.29)

For simplification, a new coordinate system (x, y’’, z) is introduced as shown in Figure 3.1. Here, y’’ = 0 lies at y’ =  lcu1.

On solving Eq. 3.27, the following equations are obtained:

- T =C1ⅇm1y''+C2  ⅇm2y''                                                                                                                 3.30

- ⅆTⅆy''=m1C1ⅇm1y''+ m2C2ⅇm2y''                                                                                        (3.31)

where, m1 =  -A+A2-4B2                                                                                                               (3.32)

m2 =  -A-A2-4B2                                                                                                                 (3.33)

The following boundary conditions can be applied to the p type semiconductor (y’’ = 0 to y’’ = lsc):

- The temperature at the top surface of the semiconductor will be equal to the temperature of the bottom surface of the copper plate:

Tpy''=0=Tcu1y'=lcu1#3.34

- Due to Peltier effect between copper plate and p type semiconductor, some heat flux will be lost when heat enters from bottom surface of the copper plate to the top surface of the p-type semiconductor:

kplpⅆTpⅆyat y''= 0= kculp+lnⅆTcu1ⅆyat y'= lcu1 - knlnⅆTnⅆyat y''= 0- ΠplpJy,p+ΠnlnJy,n                                                                                                                                       (3.35)

- Due to Peltier effect between copper plate and p type semiconductor, some heat flux will be added when heat exits from bottom surface of the p-type semiconductor to the top surface of the copper plater:

kplpⅆTpⅆyat y''= lsc= kculp+lnⅆTcu2ⅆyat y''= lsc- knlnⅆTnⅆyat y''= lsc- ΠplpJy,p+ΠnlnJy,n                                                                                                              (3.36)

The following boundary conditions can be applied to the n type semiconductor (y’’ = 0 to y’’ = lsc):

- The temperature at the top surface of the semiconductor will be equal to the temperature of the bottom surface of the copper plate:

Tny''=0=Tcu1y'=lcu1#3.37

- Due to Peltier effect between copper plate and p type semiconductor, some heat flux will be added when heat enters from bottom surface of the copper plate to the top surface of the p-type semiconductor:

knlnⅆTnⅆyat y''= 0= kculp+lnⅆTcu1ⅆyat y'= lcu1 - kplpⅆTpⅆyat y''= 0- ΠplpJy,p+ΠnlnJy,n                                                                                                                                       (3.38)

- Due to Peltier effect between copper plate and p type semiconductor, some heat flux will be lost when heat exits from bottom surface of the p-type semiconductor to the top surface of the copper plater:

knlnⅆTnⅆyat y''= lsc= kculp+lnⅆTcu2ⅆyat y''= lsc- kplpⅆTpⅆyat y''= lsc- ΠplpJy,p+ΠnlnJy,n                                                                                                             (3.39)

Similarly, expressions for bottomside Copper and Ceramic plates can be obtained.

## Matrix of Constants

Eqs 3.16 to 3.39, can be represented in the matrix form as:,

CX=D #3.40

where, D is the output vector = [ TH ,   0,   0,   0,   0,   TC,   0,   0,   0,   0,   0,   0,   -ρcu I2kculcu2,   -ρcu I2kculcu2 ]T

X is the variable vector = [C1,ce1, C2,ce1, C1,cu1, C2,cu1, C3,cu1, C1,p, C2,p, C1,n, C2,n, C1,cu2, C2,cu2, C3,cu2, C1,ce2,   C2,ce2]T

and C is coefficient matrix, given by the following table:

| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | -kcukce | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | lculp + ln | lp + ln | 0 | -kplpm1,pkcu-Iαpkcu | -kplpm2,pkcu-Iαpkcu | -knlnm1,nkcu+Iαnkcu | -knlnm2,nkcu+Iαnkcu | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | m1,pem1,plsc | m2,pem2,plsc | knlnkplpm1,nem1,nlsc | knlnkplpm2,nem2,nlsc | 0 | -kcukplplp+ln | -Iαnkplp + Iαpkplp | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | lcu | 1 | 0 | -kcekcu | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | lce | 1 |
| lce | 1 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | lcu22 | lcu | 1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | lcu22 | lcu | 1 | 0 | 0 | -1 | -1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | em1,plsc | em2,plsc | 0 | 0 | 0 | 0 | -1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | em1,nlsc | em2,nlsc | 0 | 0 | -1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | lcu22 | lcu | 1 | 0 | -1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

Table 2: Matrix of constants

Thus, we can obtain the variable vector as,     X=C-1D #3.41

## Current Generated

From Eq 3.41, the values of the constants are obtained in terms of i.

If all the heat energy lost in the thermoelectric module gets converted to electric energy, then the following equation can be obtained.

-kcenlp+lg+lnC1,ce1+kcenlp+lg+lnC1,ce2=i2RL #3.42

where, n = number of n-p pairs (thermocouples) in the TEG.

The above equation is numerically solved using Newton-Raphson method in order to obtain the value of current (i). Once the current value is obtained, the values of constants can be calculated. And thereafter total output power and efficiency can be calculated using the following formulas.

Power output (P) = I2RL                                                        (3.43)

and, Efficiency (η) = I2RL-kcenlp+lg+lnC1,ce1                                                   (3.44)

# Result and Discussion

For the purpose of numerical simulation, we have taken Bismuth Telluride (Bi2Te3) as the material for both p and n type semiconductors. The properties corresponding to p-type and n-type Bi2Te3 have been listed below in the following table.

| Properties | p-type Bi2Te3 | n-type Bi2Te3 |
| --- | --- | --- |
| Seebeck coefficient (α) | -190 × 10-6 V/K | 190 × 106 V/K |
| Thompson coefficient (μ) | -0.8 μV/K | 0.8 μV/K |
| Thermal conductivity (k) | 1.66 W/m.K | 1.66 W/m.K |
| Specific resistance (ρ) | 1.5 × 10-8 Ωm | 1.5 × 10-8 Ωm |
| Width (l) | 1.4 mm | 1.4 mm |

Table 3: Properties of Bi2Te3

Resistance in the external circuit (RL) is taken in the range of 50 Ohms to 250 Ohms. Temperature at the hot side is varied in the range of 400 K to 600 K and the temperature at the cold side is take in the range of 313 K to 413 K.  The other parameters required for numerical simulation have been listed below.

| Thermal conductivity of ceramic plate (kce) | 230 W/m.K | Thermal conductivity of copper plate (kcu) | 398 W/m.K |
| --- | --- | --- | --- |
| Thickness of the ceramic plate (bce) | 0.5 mm | Thickness of the copper plate (bcu) | 3 mm |
| Gap between p-type and n-type legs (bg) | 1.5 mm | Thickness of the semiconductor (bsc) | 5 mm – 15 mm |
| No. of N-P pairs (n) | 100 - 200 | Specific Resistance of copper (ρcu) | 1.68 × 10-8 Ωm |

Table 4: Value of parameters

The following studies have been performed:

- In the first parametric study, the load resistance in the external circuit has been varied from 50 Ohms to 250 Ohms and the corresponding variation in electrical power output and efficiency has been found out, which is shown in Figure 4.1. It can be seen that the output power and efficiency decrease exponentially with increase in load resistance in the external circuit. This happens because with in load resistance, current flowing through the circuit reduces, which causes output power to reduce and thus the efficiency of the TEG.

Figure 4.1: Output Power and Efficiency vs Load Resistance

- Next, the hot side temperature has been varied from 400 K to 600 K and the corresponding variation in electrical power output and efficiency has been found out, which is shown below.

Figure 4.2: Output Power and Efficiency vs Hot side Temperature

As can be seen from the diagram as the hot side temperature increases, total power output as well as efficiency increases. The main reason for that is, with increase in hot side temperature while keeping the cold side temperature constant, the heat influx increases, which increases Seebeck coefficient value, which in turn increases the current and thus the output power and efficiency.

- Now, the cold side temperature has been varied from 313 K to 413 K and the corresponding variation in electrical power output and efficiency has been plotted, which is depicted below.

Figure 4.3: Output Power and Efficiency vs Cold side Temperature

Now as the cold side temperature increases, the temperature difference between both ends of the thermoelectric generator decreases, which the potential difference developed between the both ends of the semiconductor, as result the output power decreases. Now, the heat influx also reduces with increase in temperature at the cold side of the TE module, but due to higher decrease in output power, the overall efficiency reduces.

- Next, the thickness of the semiconductor has been varied from 5 mm to 15 mm and the corresponding variation in electrical power output and efficiency has been plotted, which is shown in the Figure 4.4. With increase in semiconductor thickness, overall power output and efficiency both is seen to be increasing. It may be due to the fact that with increase in semiconductor’s height, the overall number of charge carriers increases which results in a higher potential difference between the ends.

Figure 4.4: Output Power and Efficiency vs Height of Semiconductor

- Finally, the number of N-P pairs i.e. thermoelectric modules have been varied from 100 to 200 and the corresponding variation in the electrical output power and efficiency as obtained through the numerical simulation of the model have been plotted below.

Figure 4.2: Output Power and Efficiency vs no. of n-p pairs

From the above plot, it can be seen that with increase in number of thermocouples, power output of the TEG increases, which is obvious as with n no. of thermocouples, power generated will be equal to the n times the power generated by a single thermocouple. Again, with increase in thermocouples, heat influx increases but the increase in output power is more prominent as some of the heat that could be lost, can be regained by the close spacing of the thermocouples, which increases the efficiency.

# Conclusion and Future Work

## Conclusion

This study systematically modeled and analyzed thermoelectric generators (TEGs), focusing on their potential for waste heat conversion into electrical energy. The mathematical framework incorporated key thermoelectric effects, including Seebeck, Peltier, Thomson, and Joule effects, providing a comprehensive understanding of the interactions between thermal and electrical phenomena within TEG systems. Analytical solutions were derived for each TEG component, and the model was validated through numerical simulations to evaluate the effects of design and operating parameters on power output and efficiency.

Key results include:

- Increasing the hot-side temperature or semiconductor height significantly enhances power output and efficiency.

- Higher cold-side temperatures reduce the temperature gradient, resulting in lower output and efficiency.

- Larger external load resistances negatively impact performance due to reduced current flow.

- Increasing the number of thermoelectric module pairs proportionally increases power output, demonstrating scalability.

These findings establish a robust theoretical foundation for optimizing TEG design and operation, addressing critical challenges in their application for waste heat recovery.

## Future Work

The next phase of the research will focus on expanding the analysis to various semiconductor materials to determine the optimal choice for different operating conditions. By investigating material-specific parameters such as Seebeck coefficient, thermal conductivity, and electrical resistivity, the study aims to identify semiconductors that balance efficiency and cost-effectiveness. Following material selection, experimental validation will be conducted to compare theoretical predictions with real-world performance.This work will bridge the gap between theoretical modeling and practical implementation, paving the way for broader adoption of TEGs in industrial and renewable energy applications.

# References

[1] Kousksou T, Bruel P, Jamil A, El Rhafiki T, Zeraouli Y. Energy storage: applications and challenges. Sol Energy Mat Sol C 2014;120:59e80.

[2] Harman TC, Honig JM. Thermoelectric and thermomagnetic effects and applications. New York: McGraw-Hill; 1967.

[3] Seebeck TJ. Magnetiche polarisation der metalle und erze durch temperaturdifferentz. Abh Kn Akad Wiss 1822:265e373.

[4] Huesgen T, Woias P, Kockmann N. Design and fabrication of MEMS thermoelectric generators with high temperature efficiency. Sens Actuat A-Phys 2008;145e146(1e2):423e9.

[5] Fleurial J-P. Thermoelectric power generation materials: technology and application opportunities. JOM 2009;61(4):79e85.

[6] Zebarjadi M, Esfarjani K, Dresselhaus MS, Ren ZF, Chen G. Perspectives on thermoelectrics: from fundamentals to device applications. Energy Environ Sci 2012;5:5147e62.

[7] Bubnova O, Crispin X. Towards polymer-based organic thermoelectric generators. Energy Environ Sci 2012;5(11):9345e62.

[8] Gunawan A, Lin C-H, Buttry DA, Mujica V, Taylor RA, Prasher RS, et al. Liquid thermoelectrics: review of recent and limited new data of thermogalvanic cell experiments. Nanosc Micros Therm 2013;17(4):304e23.

[9] Tasaka M. Thermal membrane potentialacross charged membranes and thermoosmosis. Pure Appl Chem 1986;58:1637e46.

[10] Janssen M, H€artel A, Van Roij R. Boosting capacitive blue-energy and desalination devices with waste heat. Phys Rev Lett 2014;113(26):268501.

[11] Sales BB, Burheim OS, Porada S, Presser V, Buisman CJ, Hamelers HV. Extraction of energy from small thermal differences near room temperature using capacitive membrane technology. Environ Sci Tech Lett 2014;1:356e60.

[12] Børset MT, Kang X, Burheim OS, Haarberg GM, Xu Q, Kjelstrup S. Seebeck coefficients of cells with lithium carbonate and gas electrodes. Electrochim Acta 2015;182:699e706.

[13] Kang X, Børset MT, Burheim OS, Haarberg GM, Xu Q, Kjelstrup S. Seebeck coefficients of cells with molten carbonates relevant to the metallurgical industry. Electrochim Acta 2015;182:342e50.

[14] D. Luo, R. Wang, W. Yu, W. Zhou, A novel optimization method for thermoelectric module used in waste heat recovery, Energy Convers. Manage. (2020), doi: 10.1016/j.enconman.2020.112645.

[15] D. Kraemer, Q. Jie, K. McEnaney, F. Cao, W. Liu, L. Weinstein, Concentrating solar thermoelectric generators with a peak efficiency of 7.4%, Nature Energy (2016), doi: 10.1038/nenergy.2016.153.

[16] A.A. Bayod-R ujula, A. Martínez-Gracia, A.D. Amo, M. Ca nada, S. Us ~ on, J. Uche, J.A. Tejero, Integration of thermoelectric generators (TEG) in Solar PVT panels, Int. Conf. Renew. Energies Power Qual. 17 (2019).

[17] M. Singh, J. Singh, P.Kuchroo Anshula, H. Bhatia, S. Bhagat, G. Sharma, E. Sidhu, Efficient autonomous solar panel and thermo-electric generator (TEG) integrated hybrid energy harvesting system, IEEE Xplore (2016), doi: 10.1109/ PIERS.2016.7734785.

[18] H. Ishaq, S. Islam, I. Dincer, B. Yilbas, Development and performance investigation of a biomass gasification based integrated system with thermoelectric generators, J. Cleaner Prod. (2020), doi: 10.1016/j.jclepro.2020.120625.

[19] Y. Sargolzaeiaval, V.P. Ramesh, T. V.Neumann, V. Misra, D. Vashaee, M. D.Dickey, M. C.Ozt € urka, Flexible thermoelectric generators for body heat harvesting €  enhanced device performance using high thermal conductivity elastomer encapsulation on liquid metal interconnects, Appl. Energy (2020), doi: 10.1016/ j.apenergy.2019.114370.

[20] M. Hyland, H. Hunter, J. Liu, E. Veety, D. Vashaee, Wearable thermoelectric generators for human body heat harvesting, Appl. Energy (2016) 518–524, doi: 10.1016/j.apenergy.2016.08.150.

[21] D.W. Auckland, R. Shuttleworth, A.C. Luff, B.P. Axcell, M. Rahman, Design of a semiconductor thermoelectric generator for remote subsea wellheads, IET 142 (1995) 65–70, doi: 10.1049/ip-epa:19951707.

[22] H. Ishaq, S. Islam, I. Dincer, B. Yilbas, Development and performance investigation of a biomass gasification based integrated system with thermoelectric generators, J. Cleaner Prod. (2020), doi: 10.1016/j.jclepro.2020.120625.

[23] Y. Sargolzaeiaval, V.P. Ramesh, T. V.Neumann, V. Misra, D. Vashaee, M. D.Dickey, M. C.Ozt € urka, Flexible thermoelectric generators for body heat harvesting €  enhanced device performance using high thermal conductivity elastomer encapsulation on liquid metal interconnects, Appl. Energy (2020), doi: 10.1016/ j.apenergy.2019.114370

[24] M. Hyland, H. Hunter, J. Liu, E. Veety, D. Vashaee, Wearable thermoelectric generators for human body heat harvesting, Appl. Energy (2016) 518–524, doi: 10.1016/j.apenergy.2016.08.150.

[25] B. Aravind, B. Khandelwal, P. Ramakrishn, S. Kumar, Towards the development of a high-power density, high efficiency, micro power generator, Appl. Energy (2020), doi: 10.1016/j.apenergy.2019.114386.

[26] Naito, M. (2018). Nanoparticle technology handbook. Elsevier.

[27] Drebushchak, V. A. (2007). The Peltier effect. Journal of Thermal Analysis and Calorimetry, 91(1), 311–315. https://doi.org/10.1007/s10973-007-8336-9

[28] Tritt, T. (2002). Thermoelectric Materials: principles, structure, properties, and applications. In Elsevier eBooks (pp. 1–11). https://doi.org/10.1016/b0-08-043152-6/01822-2

[29] Meseguer, J., Pérez-Grande, I., & Sanz-Andrés, A. (2012). Spacecraft thermal control. Elsevier.

# Appendix

## MATLAB Code for Parametric Study with respect to Length of Semiconductor

clc; clear; close all;
Power = zeros(1, 51); Efficiency = zeros(1, 51);

Parameters of the TEG Model. Bi2TE3 is taken as Semi-conductor.

syms I T_H T_C l_sc l_g n_NP;
% j = current density vector.
% T_H = Hot side temperature = 500 K
% T_C = Cold side temperature = 313 K
% l_sc = length of the semiconductor, same for both p and n type.
% l_g = distance between two legs = 1.5 mm
% n_NP = number of NP pairs = 150

% Parameters for the ceramic plate on the hot side.
syms l_ce k_ce;
% l_ce = thickness of the ceramic plate = 0.5 mm
% k_ce = thermal conductivity of ceramic = 230 W/m.K

% Parameters for the copper plate on the hot side.
syms l_cu k_cu rho_cu;
% l_cu = thickness of copper plate = 3 mm
% k_cu = thermal conductivity of copper plate = 398 W/m.K
% rho_cu = specific resistance of copper = 1.68 * 10^(-8) ohm m.

% Parameters for the n-type semiconductor.
syms l_n k_n alpha_n mu_n rho_n
% l_n = width of the n-type semiconductor = 1.4 mm
% k_n = thermal conductivity of n-type semicoductor = 1.66 W/m.K
% alpha_n = Seebeck coefficient of n-type semiconductor = 190 * 10^(-6) V/K
% mu_n = Thompson coefficient of n-type semiconductor = 0.8 * 10^(-6) V/K
% rho_n = specific resistance of n-type semiconductor = 1.5 * 10^(-5) ohm m.
A_n = ((-1) * (mu_n + alpha_n) * (I/l_n))/k_n; B_n = (rho_n * ((I/l_n) ^ 2))/k_n;
m1_n = ((-A_n) + sqrt((A_n^2) - (4 * B_n)))/2;
m2_n = ((-A_n) - sqrt((A_n^2) - (4 * B_n)))/2;

% Parameters for the p-type semiconductor.
syms l_p k_p alpha_p mu_p rho_p
% l_p = width of the p-type semiconductor = 1.4 mm
% k_p = thermal conductivity of p-type semicoductor = 1.66 W/m.K
% alpha_p = Seebeck coefficient of p-type semiconductor = -190 * 10^(-6) V/K
% mu_p = Thompson coefficient of p-type semiconductor = -0.8 * 10^(-6) V/K
% rho_p = specific resistance of p-type semiconductor = 1.5 * 10^(-5) ohm m.
A_p = ((-1) * (mu_p + alpha_p) * (I/l_p))/k_p; B_p = (rho_p * ((I/l_p) ^ 2))/k_p; % modified
m1_p = ((-A_p) + sqrt((A_p^2) - (4 * B_p)))/2;
m2_p = ((-A_p) - sqrt((A_p^2) - (4 * B_p)))/2;

% Parameters for the copper plate on the cold side.
% Same as hot side.

% Parameters for the ceramic plate on the cold side.
% Same as hot side.

% Parameters of external circuit.
syms R_L
% R_L = Load in the external circuit = 150 Ohm

TEG Model

We are going to solve an algebraic equation.

Coefficient_Matrix = [0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    1, 0, 0, ((-k_cu)/k_ce), 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, (l_cu * (l_p + l_n)), (l_p + l_n), 0, (((((-k_p) * l_p)/k_cu) * m1_p) - ((I * alpha_p)/k_cu)), (((((-k_p) * l_p)/k_cu) * m2_p) - ((I * alpha_p)/k_cu)), (((((-k_n) * l_n)/k_cu) * m1_n) + ((I * alpha_n)/k_cu)), (((((-k_n) * l_n)/k_cu) * m2_n) + ((I * alpha_n)/k_cu)), 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, (m1_p * exp(m1_p * l_sc)), (m2_p * exp(m2_p * l_sc)), ((k_n * l_n)/(k_p * l_p) * m1_n * exp(m1_n * l_sc)), ((k_n * l_n)/(k_p * l_p) * m2_n * exp(m2_n * l_sc)), 0, ((-k_cu)/(k_p * l_p) * (l_p + l_n)), (-((I * alpha_n)/(k_p * l_p)) + ((I * alpha_p)/(k_p * l_p))), 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, l_cu, 1, 0, ((-k_ce)/k_cu), 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, l_ce, 1;
    l_ce, 1, 0, 0, -1, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, -1, -1, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, 0, -1, -1, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, exp(m1_p * l_sc), exp(m2_p * l_sc), 0, 0, 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, exp(m1_n * l_sc), exp(m2_n * l_sc), 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, -1;
    0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0];
Output_Vector = [T_H; 0; 0; 0; 0; T_C; 0; 0; 0; 0; 0; 0; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu];

% Variables : C1_ce1, C2_ce1, C1_cu1, C2_cu1, C3_cu1, C1_p, C2_p, C1_n,
% C2_n, C1_cu2, C2_cu2, C3_cu2, C1_ce2, C2_ce2.
Variable_vector = inv(Coefficient_Matrix) * Output_Vector; %#ok<MINV>

% Getting the current density.
Var_eqn_LHS = ((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)) + (k_ce * n_NP * (l_p + l_g + l_n) * Variable_vector(13,1)) - ((I^2) * R_L);

counter = 1;
for Sc_length = 0.005 : 0.0002 : 0.015 % varying the load resistance from 50 to 250 ohm.

eqn_LHS = subs(Var_eqn_LHS, [R_L, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p], [150, 313, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, Sc_length, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))]);

% Solving Equation using N-R method.
I_Initial = 20; % Taking the initial value of current to be 2 amps.
d_eqn_LHS = diff(eqn_LHS, I); % Differentiating the L.H.S of the equation.

for j = 1 : 25
    I_sol = I_Initial - (double(subs(eqn_LHS, I, I_Initial))/double(subs(d_eqn_LHS, I, I_Initial)));
    I_Initial = I_sol;
end
I_sol_real = real(I_sol); % Extracting the real part.
Power(1,counter) = (I_sol_real ^ 2) * 150; % Calculating the power.
Heat_In = real(double(subs(((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)), [I, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p],[I_sol_real, 313, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, Sc_length, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))])));
Efficiency(1,counter) = (Power(1,counter) / (Heat_In)) * 100;
counter = counter + 1;
end

Plotting

subplot(2,1,1);
plot(0.005 : 0.0002 : 0.015, Power, '-b');
xlabel("Semiconductor height (m) --->");
ylabel("Output Power (W) --->");
title("Output Power vs Semiconductor height");
grid on;

subplot(2,1,2);
plot(0.005 : 0.0002 : 0.015, Efficiency, '-r');
xlabel("Semiconductor height (m) --->");
ylabel("Efficiency (%) --->");
title("Efficiency vs Semiconductor height");
grid on;

Published with MATLAB® R2022b

## MATLAB Code for Parametric Study with respect to External Circuit Load

clc; clear; close all;
Power = zeros(1, 51); Efficiency = zeros(1, 51);

Parameters of the TEG Model. Bi2TE3 is taken as Semi-conductor.

syms I T_H T_C l_sc l_g n_NP;
% j = current density vector.
% T_H = Hot side temperature = 500 K
% T_C = Cold side temperature = 313 K
% l_sc = length of the semiconductor, same for both p and n type = 7.5 mm.
% l_g = distance between two legs = 1.5 mm
% n_NP = number of NP pairs = 150

% Parameters for the ceramic plate on the hot side.
syms l_ce k_ce;
% l_ce = thickness of the ceramic plate = 0.5 mm
% k_ce = thermal conductivity of ceramic = 230 W/m.K

% Parameters for the copper plate on the hot side.
syms l_cu k_cu rho_cu;
% l_cu = thickness of copper plate = 3 mm
% k_cu = thermal conductivity of copper plate = 398 W/m.K
% rho_cu = specific resistance of copper = 1.68 * 10^(-8) ohm m.

% Parameters for the n-type semiconductor.
syms l_n k_n alpha_n mu_n rho_n
% l_n = width of the n-type semiconductor = 1.4 mm
% k_n = thermal conductivity of n-type semicoductor = 1.66 W/m.K
% alpha_n = Seebeck coefficient of n-type semiconductor = 190 * 10^(-6) V/K
% mu_n = Thompson coefficient of n-type semiconductor = 0.8 * 10^(-6) V/K
% rho_n = specific resistance of n-type semiconductor = 1.5 * 10^(-5) ohm m.
A_n = ((-1) * (mu_n + alpha_n) * (I/l_n))/k_n; B_n = (rho_n * ((I/l_n) ^ 2))/k_n;
m1_n = ((-A_n) + sqrt((A_n^2) - (4 * B_n)))/2;
m2_n = ((-A_n) - sqrt((A_n^2) - (4 * B_n)))/2;

% Parameters for the p-type semiconductor.
syms l_p k_p alpha_p mu_p rho_p
% l_p = width of the p-type semiconductor = 1.4 mm
% k_p = thermal conductivity of p-type semicoductor = 1.66 W/m.K
% alpha_p = Seebeck coefficient of p-type semiconductor = -190 * 10^(-6) V/K
% mu_p = Thompson coefficient of p-type semiconductor = -0.8 * 10^(-6) V/K
% rho_p = specific resistance of p-type semiconductor = 1.5 * 10^(-5) ohm m.
A_p = ((-1) * (mu_p + alpha_p) * (I/l_p))/k_p; B_p = (rho_p * ((I/l_p) ^ 2))/k_p; % modified
m1_p = ((-A_p) + sqrt((A_p^2) - (4 * B_p)))/2;
m2_p = ((-A_p) - sqrt((A_p^2) - (4 * B_p)))/2;

% Parameters for the copper plate on the cold side.
% Same as hot side.

% Parameters for the ceramic plate on the cold side.
% Same as hot side.

% Parameters of external circuit.
syms R_L
% R_L = Load in the external circuit.

TEG Model

We are going to solve an algebraic equation.

Coefficient_Matrix = [0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    1, 0, 0, ((-k_cu)/k_ce), 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, (l_cu * (l_p + l_n)), (l_p + l_n), 0, (((((-k_p) * l_p)/k_cu) * m1_p) - ((I * alpha_p)/k_cu)), (((((-k_p) * l_p)/k_cu) * m2_p) - ((I * alpha_p)/k_cu)), (((((-k_n) * l_n)/k_cu) * m1_n) + ((I * alpha_n)/k_cu)), (((((-k_n) * l_n)/k_cu) * m2_n) + ((I * alpha_n)/k_cu)), 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, (m1_p * exp(m1_p * l_sc)), (m2_p * exp(m2_p * l_sc)), ((k_n * l_n)/(k_p * l_p) * m1_n * exp(m1_n * l_sc)), ((k_n * l_n)/(k_p * l_p) * m2_n * exp(m2_n * l_sc)), 0, ((-k_cu)/(k_p * l_p) * (l_p + l_n)), (-((I * alpha_n)/(k_p * l_p)) + ((I * alpha_p)/(k_p * l_p))), 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, l_cu, 1, 0, ((-k_ce)/k_cu), 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, l_ce, 1;
    l_ce, 1, 0, 0, -1, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, -1, -1, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, 0, -1, -1, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, exp(m1_p * l_sc), exp(m2_p * l_sc), 0, 0, 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, exp(m1_n * l_sc), exp(m2_n * l_sc), 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, -1;
    0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0];
Output_Vector = [T_H; 0; 0; 0; 0; T_C; 0; 0; 0; 0; 0; 0; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu];

% Variables : C1_ce1, C2_ce1, C1_cu1, C2_cu1, C3_cu1, C1_p, C2_p, C1_n,
% C2_n, C1_cu2, C2_cu2, C3_cu2, C1_ce2, C2_ce2.
Variable_vector = inv(Coefficient_Matrix) * Output_Vector; %#ok<MINV>

% Getting the current density.
Var_eqn_LHS = ((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)) + (k_ce * n_NP * (l_p + l_g + l_n) * Variable_vector(13,1)) - ((I^2) * R_L);

counter = 1;
for Load_R = 1 : 1 : 51 % varying the load resistance from 50 to 250 ohm.

eqn_LHS = subs(Var_eqn_LHS, [R_L, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p], [Load_R, 313, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))]);

% Solving Equation using N-R method.
I_Initial = 20; % Taking the initial value of current to be 2 amps.
d_eqn_LHS = diff(eqn_LHS, I); % Differentiating the L.H.S of the equation.

for j = 1 : 25
    I_sol = I_Initial - (double(subs(eqn_LHS, I, I_Initial))/double(subs(d_eqn_LHS, I, I_Initial)));
    I_Initial = I_sol;
end
I_sol_real = real(I_sol); % Extracting the real part.
Power(1,counter) = (I_sol_real ^ 2) * Load_R; % Calculating the power.
Heat_In = real(double(subs(((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)), [I, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p],[I_sol_real, 313, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))])));
Efficiency(1,counter) = (Power(1,counter) / (Heat_In)) * 100;
counter = counter + 1;
end

Plotting

subplot(2,1,1);
plot(1 : 1 : 51, Power, '-b');
xlabel("Load Resistance (Ohm) --->");
ylabel("Output Power (W) --->");
title("Output Power vs Load Resistance");
grid on;

subplot(2,1,2);
plot(1 : 1 : 51, Efficiency, '-r');
xlabel("Load Resistance (Ohm) --->");
ylabel("Efficiency (%) --->");
title("Efficiency vs Load Resistance");
grid on;

Published with MATLAB® R2022b

## MATLAB Code for Parametric Study with respect to Number of Semiconductor pairs

clc; clear; close all;
Power = zeros(1, 51); Efficiency = zeros(1, 51);

Parameters of the TEG Model. Bi2TE3 is taken as Semi-conductor.

syms I T_H T_C l_sc l_g n_NP;
% j = current density vector.
% T_H = Hot side temperature = 500 K
% T_C = Cold side temperature = 313 K
% l_sc = length of the semiconductor, same for both p and n type = 7.5 mm
% l_g = distance between two legs = 1.5 mm
% n_NP = number of NP pairs.

% Parameters for the ceramic plate on the hot side.
syms l_ce k_ce;
% l_ce = thickness of the ceramic plate = 0.5 mm
% k_ce = thermal conductivity of ceramic = 230 W/m.K

% Parameters for the copper plate on the hot side.
syms l_cu k_cu rho_cu;
% l_cu = thickness of copper plate = 3 mm
% k_cu = thermal conductivity of copper plate = 398 W/m.K
% rho_cu = specific resistance of copper = 1.68 * 10^(-8) ohm m.

% Parameters for the n-type semiconductor.
syms l_n k_n alpha_n mu_n rho_n
% l_n = width of the n-type semiconductor = 1.4 mm
% k_n = thermal conductivity of n-type semicoductor = 1.66 W/m.K
% alpha_n = Seebeck coefficient of n-type semiconductor = 190 * 10^(-6) V/K
% mu_n = Thompson coefficient of n-type semiconductor = 0.8 * 10^(-6) V/K
% rho_n = specific resistance of n-type semiconductor = 1.5 * 10^(-5) ohm m.
A_n = ((-1) * (mu_n + alpha_n) * (I/l_n))/k_n; B_n = (rho_n * ((I/l_n) ^ 2))/k_n;
m1_n = ((-A_n) + sqrt((A_n^2) - (4 * B_n)))/2;
m2_n = ((-A_n) - sqrt((A_n^2) - (4 * B_n)))/2;

% Parameters for the p-type semiconductor.
syms l_p k_p alpha_p mu_p rho_p
% l_p = width of the p-type semiconductor = 1.4 mm
% k_p = thermal conductivity of p-type semicoductor = 1.66 W/m.K
% alpha_p = Seebeck coefficient of p-type semiconductor = -190 * 10^(-6) V/K
% mu_p = Thompson coefficient of p-type semiconductor = -0.8 * 10^(-6) V/K
% rho_p = specific resistance of p-type semiconductor = 1.5 * 10^(-5) ohm m.
A_p = ((-1) * (mu_p + alpha_p) * (I/l_p))/k_p; B_p = (rho_p * ((I/l_p) ^ 2))/k_p; % modified
m1_p = ((-A_p) + sqrt((A_p^2) - (4 * B_p)))/2;
m2_p = ((-A_p) - sqrt((A_p^2) - (4 * B_p)))/2;

% Parameters for the copper plate on the cold side.
% Same as hot side.

% Parameters for the ceramic plate on the cold side.
% Same as hot side.

% Parameters of external circuit.
syms R_L
% R_L = Load in the external circuit = 150 Ohm

TEG Model

We are going to solve an algebraic equation.

Coefficient_Matrix = [0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    1, 0, 0, ((-k_cu)/k_ce), 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, (l_cu * (l_p + l_n)), (l_p + l_n), 0, (((((-k_p) * l_p)/k_cu) * m1_p) - ((I * alpha_p)/k_cu)), (((((-k_p) * l_p)/k_cu) * m2_p) - ((I * alpha_p)/k_cu)), (((((-k_n) * l_n)/k_cu) * m1_n) + ((I * alpha_n)/k_cu)), (((((-k_n) * l_n)/k_cu) * m2_n) + ((I * alpha_n)/k_cu)), 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, (m1_p * exp(m1_p * l_sc)), (m2_p * exp(m2_p * l_sc)), ((k_n * l_n)/(k_p * l_p) * m1_n * exp(m1_n * l_sc)), ((k_n * l_n)/(k_p * l_p) * m2_n * exp(m2_n * l_sc)), 0, ((-k_cu)/(k_p * l_p) * (l_p + l_n)), (-((I * alpha_n)/(k_p * l_p)) + ((I * alpha_p)/(k_p * l_p))), 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, l_cu, 1, 0, ((-k_ce)/k_cu), 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, l_ce, 1;
    l_ce, 1, 0, 0, -1, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, -1, -1, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, 0, -1, -1, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, exp(m1_p * l_sc), exp(m2_p * l_sc), 0, 0, 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, exp(m1_n * l_sc), exp(m2_n * l_sc), 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, -1;
    0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0];
Output_Vector = [T_H; 0; 0; 0; 0; T_C; 0; 0; 0; 0; 0; 0; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu];

% Variables : C1_ce1, C2_ce1, C1_cu1, C2_cu1, C3_cu1, C1_p, C2_p, C1_n,
% C2_n, C1_cu2, C2_cu2, C3_cu2, C1_ce2, C2_ce2.
Variable_vector = inv(Coefficient_Matrix) * Output_Vector; %#ok<MINV>

% Getting the current density.
Var_eqn_LHS = ((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)) + (k_ce * n_NP * (l_p + l_g + l_n) * Variable_vector(13,1)) - ((I^2) * R_L);

counter = 1;
for no_NP = 100 : 2 : 200 % varying the load resistance from 50 to 250 ohm.

eqn_LHS = subs(Var_eqn_LHS, [R_L, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p], [150, 313, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), no_NP, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))]);

% Solving Equation using N-R method.
I_Initial = 20; % Taking the initial value of current to be 2 amps.
d_eqn_LHS = diff(eqn_LHS, I); % Differentiating the L.H.S of the equation.

for j = 1 : 25
    I_sol = I_Initial - (double(subs(eqn_LHS, I, I_Initial))/double(subs(d_eqn_LHS, I, I_Initial)));
    I_Initial = I_sol;
end
I_sol_real = real(I_sol); % Extracting the real part.
Power(1,counter) = (I_sol_real ^ 2) * 150; % Calculating the power.
Heat_In = real(double(subs(((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)), [I, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p],[I_sol_real, 313, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), no_NP, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))])));
Efficiency(1,counter) = (Power(1,counter) / (Heat_In)) * 100;
counter = counter + 1;
end

Plotting

subplot(2,1,1);
plot(100 : 2 : 200, Power, '-b');
xlabel("No. of N-P pairs --->");
ylabel("Output Power (W) --->");
title("Output Power vs No. of N-P pairs");
grid on;

subplot(2,1,2);
plot(100 : 2 : 200, Efficiency, '-r');
xlabel("No. of N-P pairs --->");
ylabel("Efficiency (%) --->");
title("Efficiency vs No. of N-P pairs");
grid on;

Published with MATLAB® R2022b

## MATLAB Code for Parametric Study with respect to Cold Side Temperature

clc; clear; close all;
Power = zeros(1, 51); Efficiency = zeros(1, 51);

Parameters of the TEG Model. Bi2TE3 is taken as Semi-conductor.

syms I T_H T_C l_sc l_g n_NP;
% j = current density vector.
% T_H = Hot side temperature = 500 K
% T_C = Cold side temperature.
% l_sc = length of the semiconductor, same for both p and n type = 7.5 mm.
% l_g = distance between two legs = 1.5 mm
% n_NP = number of NP pairs = 150

% Parameters for the ceramic plate on the hot side.
syms l_ce k_ce;
% l_ce = thickness of the ceramic plate = 0.5 mm
% k_ce = thermal conductivity of ceramic = 230 W/m.K

% Parameters for the copper plate on the hot side.
syms l_cu k_cu rho_cu;
% l_cu = thickness of copper plate = 3 mm
% k_cu = thermal conductivity of copper plate = 398 W/m.K
% rho_cu = specific resistance of copper = 1.68 * 10^(-8) ohm m.

% Parameters for the n-type semiconductor.
syms l_n k_n alpha_n mu_n rho_n
% l_n = width of the n-type semiconductor = 1.4 mm
% k_n = thermal conductivity of n-type semicoductor = 1.66 W/m.K
% alpha_n = Seebeck coefficient of n-type semiconductor = 190 * 10^(-6) V/K
% mu_n = Thompson coefficient of n-type semiconductor = 0.8 * 10^(-6) V/K
% rho_n = specific resistance of n-type semiconductor = 1.5 * 10^(-5) ohm m.
A_n = ((-1) * (mu_n + alpha_n) * (I/l_n))/k_n; B_n = (rho_n * ((I/l_n) ^ 2))/k_n;
m1_n = ((-A_n) + sqrt((A_n^2) - (4 * B_n)))/2;
m2_n = ((-A_n) - sqrt((A_n^2) - (4 * B_n)))/2;

% Parameters for the p-type semiconductor.
syms l_p k_p alpha_p mu_p rho_p
% l_p = width of the p-type semiconductor = 1.4 mm
% k_p = thermal conductivity of p-type semicoductor = 1.66 W/m.K
% alpha_p = Seebeck coefficient of p-type semiconductor = -190 * 10^(-6) V/K
% mu_p = Thompson coefficient of p-type semiconductor = -0.8 * 10^(-6) V/K
% rho_p = specific resistance of p-type semiconductor = 1.5 * 10^(-5) ohm m.
A_p = ((-1) * (mu_p + alpha_p) * (I/l_p))/k_p; B_p = (rho_p * ((I/l_p) ^ 2))/k_p; % modified
m1_p = ((-A_p) + sqrt((A_p^2) - (4 * B_p)))/2;
m2_p = ((-A_p) - sqrt((A_p^2) - (4 * B_p)))/2;

% Parameters for the copper plate on the cold side.
% Same as hot side.

% Parameters for the ceramic plate on the cold side.
% Same as hot side.

% Parameters of external circuit.
syms R_L
% R_L = Load in the external circuit = 150 Ohm

TEG Model

We are going to solve an algebraic equation.

Coefficient_Matrix = [0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    1, 0, 0, ((-k_cu)/k_ce), 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, (l_cu * (l_p + l_n)), (l_p + l_n), 0, (((((-k_p) * l_p)/k_cu) * m1_p) - ((I * alpha_p)/k_cu)), (((((-k_p) * l_p)/k_cu) * m2_p) - ((I * alpha_p)/k_cu)), (((((-k_n) * l_n)/k_cu) * m1_n) + ((I * alpha_n)/k_cu)), (((((-k_n) * l_n)/k_cu) * m2_n) + ((I * alpha_n)/k_cu)), 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, (m1_p * exp(m1_p * l_sc)), (m2_p * exp(m2_p * l_sc)), ((k_n * l_n)/(k_p * l_p) * m1_n * exp(m1_n * l_sc)), ((k_n * l_n)/(k_p * l_p) * m2_n * exp(m2_n * l_sc)), 0, ((-k_cu)/(k_p * l_p) * (l_p + l_n)), (-((I * alpha_n)/(k_p * l_p)) + ((I * alpha_p)/(k_p * l_p))), 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, l_cu, 1, 0, ((-k_ce)/k_cu), 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, l_ce, 1;
    l_ce, 1, 0, 0, -1, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, -1, -1, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, 0, -1, -1, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, exp(m1_p * l_sc), exp(m2_p * l_sc), 0, 0, 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, exp(m1_n * l_sc), exp(m2_n * l_sc), 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, -1;
    0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0];
Output_Vector = [T_H; 0; 0; 0; 0; T_C; 0; 0; 0; 0; 0; 0; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu];

% Variables : C1_ce1, C2_ce1, C1_cu1, C2_cu1, C3_cu1, C1_p, C2_p, C1_n,
% C2_n, C1_cu2, C2_cu2, C3_cu2, C1_ce2, C2_ce2.
Variable_vector = inv(Coefficient_Matrix) * Output_Vector; %#ok<MINV>

% Getting the current density.
Var_eqn_LHS = ((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)) + (k_ce * n_NP * (l_p + l_g + l_n) * Variable_vector(13,1)) - ((I^2) * R_L);

counter = 1;
for Cold_Side_Temp = 313 : 2 : 413 % varying the load resistance from 50 to 250 ohm.

eqn_LHS = subs(Var_eqn_LHS, [R_L, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p], [150, Cold_Side_Temp, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))]);

% Solving Equation using N-R method.
I_Initial = 20; % Taking the initial value of current to be 2 amps.
d_eqn_LHS = diff(eqn_LHS, I); % Differentiating the L.H.S of the equation.

for j = 1 : 25
    I_sol = I_Initial - (double(subs(eqn_LHS, I, I_Initial))/double(subs(d_eqn_LHS, I, I_Initial)));
    I_Initial = I_sol;
end
I_sol_real = real(I_sol); % Extracting the real part.
Power(1,counter) = (I_sol_real ^ 2) * 150; % Calculating the power.
Heat_In = real(double(subs(((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)), [I, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p],[I_sol_real, Cold_Side_Temp, 500, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))])));
Efficiency(1,counter) = (Power(1,counter) / (Heat_In)) * 100;
counter = counter + 1;
end

Plotting

subplot(2,1,1);
plot(313 : 2 : 413, Power, '-b');
xlabel("Cold side temperature (K) --->");
ylabel("Output Power (W) --->");
title("Output Power vs Cold side temperature");
grid on;

subplot(2,1,2);
plot(313 : 2 : 413, Efficiency, '-r');
xlabel("Cold side temperature (K) --->");
ylabel("Efficiency (%) --->");
title("Efficiency vs Cold side temperature");
grid on;

Published with MATLAB® R2022b

## MATLAB Code for Parametric Study with respect to Hot Side Temperature

clc; clear; close all;
Power = zeros(1, 51); Efficiency = zeros(1, 51);

Parameters of the TEG Model. Bi2TE3 is taken as Semi-conductor.

syms I T_H T_C l_sc l_g n_NP;
% j = current density vector.
% T_H = Hot side temperature.
% T_C = Cold side temperature = 313 K
% l_sc = length of the semiconductor, same for both p and n type = 7.5 mm.
% l_g = distance between two legs = 1.5 mm
% n_NP = number of NP pairs = 150

% Parameters for the ceramic plate on the hot side.
syms l_ce k_ce;
% l_ce = thickness of the ceramic plate = 0.5 mm
% k_ce = thermal conductivity of ceramic = 230 W/m.K

% Parameters for the copper plate on the hot side.
syms l_cu k_cu rho_cu;
% l_cu = thickness of copper plate = 3 mm
% k_cu = thermal conductivity of copper plate = 398 W/m.K
% rho_cu = specific resistance of copper = 1.68 * 10^(-8) ohm m.

% Parameters for the n-type semiconductor.
syms l_n k_n alpha_n mu_n rho_n
% l_n = width of the n-type semiconductor = 1.4 mm
% k_n = thermal conductivity of n-type semicoductor = 1.66 W/m.K
% alpha_n = Seebeck coefficient of n-type semiconductor = 190 * 10^(-6) V/K
% mu_n = Thompson coefficient of n-type semiconductor = 0.8 * 10^(-6) V/K
% rho_n = specific resistance of n-type semiconductor = 1.5 * 10^(-5) ohm m.
A_n = ((-1) * (mu_n + alpha_n) * (I/l_n))/k_n; B_n = (rho_n * ((I/l_n) ^ 2))/k_n;
m1_n = ((-A_n) + sqrt((A_n^2) - (4 * B_n)))/2;
m2_n = ((-A_n) - sqrt((A_n^2) - (4 * B_n)))/2;

% Parameters for the p-type semiconductor.
syms l_p k_p alpha_p mu_p rho_p
% l_p = width of the p-type semiconductor = 1.4 mm
% k_p = thermal conductivity of p-type semicoductor = 1.66 W/m.K
% alpha_p = Seebeck coefficient of p-type semiconductor = -190 * 10^(-6) V/K
% mu_p = Thompson coefficient of p-type semiconductor = -0.8 * 10^(-6) V/K
% rho_p = specific resistance of p-type semiconductor = 1.5 * 10^(-5) ohm m.
A_p = ((-1) * (mu_p + alpha_p) * (I/l_p))/k_p; B_p = (rho_p * ((I/l_p) ^ 2))/k_p; % modified
m1_p = ((-A_p) + sqrt((A_p^2) - (4 * B_p)))/2;
m2_p = ((-A_p) - sqrt((A_p^2) - (4 * B_p)))/2;

% Parameters for the copper plate on the cold side.
% Same as hot side.

% Parameters for the ceramic plate on the cold side.
% Same as hot side.

% Parameters of external circuit.
syms R_L
% R_L = Load in the external circuit = 150 Ohm

TEG Model

We are going to solve an algebraic equation.

Coefficient_Matrix = [0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    1, 0, 0, ((-k_cu)/k_ce), 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, (l_cu * (l_p + l_n)), (l_p + l_n), 0, (((((-k_p) * l_p)/k_cu) * m1_p) - ((I * alpha_p)/k_cu)), (((((-k_p) * l_p)/k_cu) * m2_p) - ((I * alpha_p)/k_cu)), (((((-k_n) * l_n)/k_cu) * m1_n) + ((I * alpha_n)/k_cu)), (((((-k_n) * l_n)/k_cu) * m2_n) + ((I * alpha_n)/k_cu)), 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, (m1_p * exp(m1_p * l_sc)), (m2_p * exp(m2_p * l_sc)), ((k_n * l_n)/(k_p * l_p) * m1_n * exp(m1_n * l_sc)), ((k_n * l_n)/(k_p * l_p) * m2_n * exp(m2_n * l_sc)), 0, ((-k_cu)/(k_p * l_p) * (l_p + l_n)), (-((I * alpha_n)/(k_p * l_p)) + ((I * alpha_p)/(k_p * l_p))), 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, l_cu, 1, 0, ((-k_ce)/k_cu), 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, l_ce, 1;
    l_ce, 1, 0, 0, -1, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, -1, -1, 0, 0, 0, 0, 0, 0, 0;
    0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, 0, -1, -1, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, exp(m1_p * l_sc), exp(m2_p * l_sc), 0, 0, 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, exp(m1_n * l_sc), exp(m2_n * l_sc), 0, 0, -1, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, ((l_cu) ^ 2)/2, l_cu, 1, 0, -1;
    0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0;
    0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0];
Output_Vector = [T_H; 0; 0; 0; 0; T_C; 0; 0; 0; 0; 0; 0; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu; ((-rho_cu) * ((I/l_cu) ^ 2)) / k_cu];

% Variables : C1_ce1, C2_ce1, C1_cu1, C2_cu1, C3_cu1, C1_p, C2_p, C1_n,
% C2_n, C1_cu2, C2_cu2, C3_cu2, C1_ce2, C2_ce2.
Variable_vector = inv(Coefficient_Matrix) * Output_Vector; %#ok<MINV>

% Getting the current density.
Var_eqn_LHS = ((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)) + (k_ce * n_NP * (l_p + l_g + l_n) * Variable_vector(13,1)) - ((I^2) * R_L);

counter = 1;
for Hot_Side_Temp = 400 : 8 : 800 % varying the load resistance from 50 to 250 ohm.

eqn_LHS = subs(Var_eqn_LHS, [R_L, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p], [150, 313, Hot_Side_Temp, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))]);

% Solving Equation using N-R method.
I_Initial = 20; % Taking the initial value of current to be 2 amps.
d_eqn_LHS = diff(eqn_LHS, I); % Differentiating the L.H.S of the equation.

for j = 1 : 25
    I_sol = I_Initial - (double(subs(eqn_LHS, I, I_Initial))/double(subs(d_eqn_LHS, I, I_Initial)));
    I_Initial = I_sol;
end
I_sol_real = real(I_sol); % Extracting the real part.
Power(1,counter) = (I_sol_real ^ 2) * 150; % Calculating the power.
Heat_In = real(double(subs(((-k_ce) * n_NP * (l_p + l_g + l_n) * Variable_vector(1,1)), [I, T_C, T_H, alpha_n, alpha_p, k_ce, k_cu, k_n, k_p, l_ce, l_cu, l_g, l_n, l_p, l_sc, mu_n, mu_p, n_NP, rho_cu, rho_n, rho_p],[I_sol_real, 313, Hot_Side_Temp, 190 * (10 ^ (-6)), (-190) * (10 ^ (-6)), 230, 398, 1.66, 1.66, 0.0005, 0.003, 0.0015, 0.0014, 0.0014, 0.0075, 0.8 * (10 ^ (-6)), (-0.8) * (10 ^ (-6)), 150, 1.68 * (10 ^ (-8)), 1.5 * (10 ^ (-8)), 1.5 * (10 ^ (-8))])));
Efficiency(1,counter) = (Power(1,counter) / (Heat_In)) * 100;
counter = counter + 1;
end

Plotting

subplot(2,1,1);
plot(400 : 8 : 800, Power, '-b');
xlabel("Hot side temperature (K) --->");
ylabel("Output Power (W) --->");
title("Output Power vs Hot side temperature");
grid on;

subplot(2,1,2);
plot(400 : 8 : 800, Efficiency, '-r');
xlabel("Hot side temperature (K) --->");
ylabel("Efficiency (%) --->");
title("Efficiency vs Hot side temperature");
grid on;

Published with MATLAB® R2022b